Name ____________________________ Group ________

Two wells located at the distance of 170 m in the direction of the groundwater flow are intersected impermeable clays overlying a water-bearing sands. The thickness of water-bearing sands is 8.3 m. Using the data in the table, plot a schematic hydrogeological cross section, fill in the gaps in the table and determine the unity flow discharge (discharge of the flow of 1 m wide). Point out the direction of groundwater flow on the hydrogeological cross section.

| Results of measurements | № of well |  | № of well |  | № of well |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 1 | 2 | 1 | 2 |
|  | Variants |  |  |  |  |  |
|  | 1 |  | 2 |  | 3 |  |
| Absolute elevation of, m: |  |  |  |  |  |  |
| well head | 66,2 | 64,9 | 27,1 | 27,8 | 53,6 | 56,2 |
| bottom of upper aquitard | 53,8 | 51,5 |  |  |  |  |
| top of underlying aquitard |  |  | 7,0 | 10,6 | 33,7 | 31,0 |
| piezometric level (standing head-water level) | 64,7 | 62,3 | 26,4 | 28,5 |  |  |
| Head (Н) above the bottom of the upper aquitard, m |  |  |  |  | 12,2 | 9,4 |
| Filtration coefficient kф, m/day | 21,4 | 21,4 | 15,6 | 15,6 | 23,1 | 23,1 |

Schematic hydrogeological cross section:

Calculations: